PFOC Memo# 12

Issued: April 11, 2005

Subject: Splice Closure and Optical Terminal Testing with Regard to Field Installed Cables. 

Background;

Verizon purchases a variety of Optical Fiber Cables for distribution and drop purposes. These cables must reliably interoperate with each other when routed through various types of optical closures and terminals used in Verizon's FTTP network. The integrity of the assembled closure or terminal is dependent on the physical size of the cables. In addition, it is not feasible to specify which cable will be available to Verizon installers at any given time. Qualification testing of optical closures and terminals must therefore be executed in a manner that exercises all probable combinations of both distribution and drop cables used by Verizon.

Requirement:

1.  PFOC Qualification of terminals and closures must be conducted using distribution and drop cables (excluding those drop cables that are connectorized) with the minimum and maximum dimensions of each cable type from all manufacturers specified in the Attachment . The maximum distribution cable dimension that must be used is to be determined by the maximum fiber capacity of the closure or terminal under test. The maximum drop cable dimension that must be used is to be determined by the sealing mechanism or grommet specified by the closure or terminal manufacturer for that particular drop cable cross-section. A supplier/cable-dimension test matrix that exercises all combinations of both distribution and drop cables used by Verizon shall be developed  as part of the qualification test plan. 

2. The minimum and maximum cable and drop that the unit under test can accommodate must be tested for each type cable manufactured by each supplier used by Verizon. 

3. Verizon recognizes that there may be a sub-set of the above cables that may adequately exercise the clamping mechanism and the cable entry system of the product being evaluated. Therefore, the ITL shall provide Verizon with the justification on which any reduced testing is based. This justification shall be based on a design analysis of the product and the attached cables. In all cases of test reduction, the ITL and the product supplier are obligated to ensure that the product is sufficiently tested for trouble free operation with any of the cables that Verizon uses and which the product is designed to support. 

Further work by Tony Ali on this subject will formalize item #3 with the Cable Manufactures. It is planned that this will be completed by July 1, 2005.  He will report at the next PFOC meeting. In the interim use the guidelines as provided above. 

Applicability:

The requirements in this document are applicable to products such as Splice Closures, Optical Terminals, Pedestals, etc, where an optical cable is installed into  the product. The applicable Generic Requirements documents include GR-771, GR-950 and GR-2898.

Effective Date:

The requirements in this document are effective immediately.

 [image: image1.wmf]Cable Dimensions.xls


Tom Amatulli - DMTS

ITL Program Manager - PFOC

Systems Integration & Testing

1095 Ave of Americas - 17 floor

NY, NY 10036

Tel:212-395-5681

 Email: thomas.p.amatulli@verizon.com

