PFOC Memo # 27 - Rev # 1

Issued: July 27, 2005 

Subject: GRs: 1209 and 1221 Splitters and other Passive Optical Components, Complete Program Test Punch Lists and Modified Test Programs/Punch Lists for Specific Scenarios 

Reason for Revision: Change in test procedure during the GR-1209 Temperature and Humidity Cycle required for "plug and play" splitters.  

The Plug and Play type splitter must be tested in a configuration that uses the actual complete FDH or a fixture that simulates the FDH receptacle shelf for the splitter.  When performing the GR-1209 Temperature and Humidity Cycle the following procedural steps are required:


At 23 C, do 10 insert/remove cycles, do IL/RL, then 42 cycles of Temp/Hum per GR-1209, do IL/RL then at 40 C, do 25 insert/remove cycles, do IL/RL, then at -18 C, do 25 insert/remove cycles, do IL/RL then at 23 C do 10 insert/remove cycles, do IL/RL

Issued By:

Tom Amatulli - DMTS

ITL Program Manager - PFOC

Systems Integration & Testing

500 Westchester Ave, Room: 1A4

White Plains, New York

Tel: 914-644-8767

 Email: thomas.p.amatulli@verizon.com
